HOME AUTOMATION USING GOOGLE ASSISTANT

In this project we are going to control our devices with voice commands using Google assistant.

- Installing required libraries

Install these libraries by typing the commands in terminal

pip install requests

sudo apt-get install build-essential python-dev

- Setting up Google Assistant and linking it to ThingSpeak Channel

Step 1: Create account on IFTTT website and click on your ID.

Step 2:  Now click on “New Applet”

Step 3:  Click on “+this”.

Step 4:  Choose Google Assistant from the list.

Step 5:  Click on “Say a simple phrase” applet.

Step 6:  Fill the following fields and click “Create trigger”.

Step 7:  Click on “+that”.

Step 8:  Search for webhooks and select it.

Step 8:  Choose action “Make a web request”.

Step 9:  Here we have to fill the ThingSpeak Channel.

Step 10:  Goto Thingspeak.com, sign in to your account and create a new channel.

Step 11: Give this channel a name.

Step 12:  Click on “API” keys.

Step 13:  Copy the “Update channel feed” link. Make sure you don’t copy “GET” prefix written in front of it.

Step 14:  Paste the URL here and change the last values according to what data you want to send. The same data will be updated on ThingSpeak channel.

For eg: 	you can send “1” as a signal to turn on LED/Light.

And you can send “0” to turn it off.

Make sure you select json from the Method drop down menu

Select content type as application/json

Now click on “Create action”.

Step 15:  Click on “Finish”.

Step 16:  Click on “check now”.

- Code to Fetch data from ThingSpeak Channel

import json

import requests

import time

import RPi.GPIO as GPIO

led = 3

GPIO.setmode(GPIO.BOARD)

GPIO.setup(led,GPIO.OUT)

myserverURL = "https://www...... enter your URL here"

while(True):

data=requests.get(myserverURL)

if data.status_code!=200:

print("error");

try:

data=json.loads(data.text)

except:

data=None;

if data!=None:

value =data["feeds"][0]["field1"]

print("value = %s" % (value))

if value=='1':

GPIO.output(led,True)

if value=='0':

GPIO.output(led,False)

time.sleep(0.01)